Application of Response Surface Methodology for Enhanced Electrochemical Process for the Removal of Phenol from Aqueous Solution using Graphite Electrodes

Hariraj Singh1 and Brijesh Kumar Mishra1* and Aditya Prakash Yadav2

1Department of Environmental Science and Engineering, Indian Institute of Technology (ISM), Dhanbad, Email: hariraj15288@gmail.com

2Department of Mechanical Engineering, School of Management Sciences, Lucknow,

Email: aditya719@gmail.com

Abstract

The aim of the present work was to investigate the removal of carbolic acid (phenol) from a synthetic solution by the enhanced electrochemical oxidation process using graphite electrodes. Central composite design (CCD) and Box Behnken Design (BBD) under Response Surface Methodology (RSM) tool were used to investigate the effects of major operating variables viz. Current density (mA/cm2): (2.27 to 4.54), pH:  (5.5 to 7.5) and electrolysis time (min): (30 to 90). The predicted values of BBD responses obtained using RSM were  more significant than the CCD model in terms of reaction time, whereas under the desirability test  CCD model was found more appropriate in terms of phenol removal and power consumption. The optimal result shows that the CCD model predicted and experimental values of phenol removal and power consumption are 92.87 %;  0.866 kWh/m3  and 86.34 %; 1.12 kWh/m3 respectively under optimized variable conditions, current density: 2.78 mA/cm2, pH: 6.98 and electrolysis time: 88.02 minutes at high desirability level.

Keywords: Electrolysis time, CCD model, BBD response.