Nanostructured copper ferrites as humidity sensor

Richa Srivastava

University Institute of Engineering and Technology

BabasahebBhimraoAmbedkar University, Lucknow-226025

Corresponding author Tel: (+91) 9415313393; Email: richadolly@rediffmail.com

Nanostructured Copper Ferrite (sample B1 and B2) were synthesized by chemical precipitation methods using two different precipitating agent i.e. sodium hydroxide and ammonium hydroxide respectively. X- ray diffraction proved the formation of copper ferrites. Crystallite sizes of the materials B1 and B2 were 86 and 14 nm respectively. Surface morphology of the sample B2 reveals that it has more adsorption sites in comparison to B1. Further the Pellets, thick and thin film of materials B1 and B2 were prepared and investigated with the exposition of humidity. Our humidity sensing properties illustrate that thin film prepared from material B2 was most sensitive among all having maximum average sensitivity 46.2 MΩ/%RH. Best sensitivity, less hysteresis and good reproducibility identifies that fabricated humidity sensor (B2) is promising and challenging.

Keywords:Humidity, sensitivity, nanomaterials, morphology