MODIFICATION OF UHPC USING INDUSTRIAL WASTE AND OTHER TYPES OF MINERAL ADMIXTURES WITH THE HELP OF STEEL FIBER

Vishavjeet                                                                                  Neelesh Kumar Singh

Assistant Professor                                                                    Assistant Professor

BSSITM, LUCKNOW                                                              SRMGPC, LUCKNOW

Abstract

The Ultra-High Performance Fibre-Reinforced Concrete is a relatively new type of Construction material, which is a combination of various types of admixtures, industrial waste and steel fibers. UHPC, or Ultra High Performance Concrete, is a class of concrete defined by its exceptionally high strength and durability .The compressive strength of UHPC reaches beyond 150 MPa, which help to allow the construction of sustainable and economic structures with an extraordinarily slim design. In general the aim is to achieve such high strength with the help of industrial wastes keeping the cement content under permissible limits. The aim herein is to develop a concrete mix incorporating silica fume, nano silica, rice husk ash and ground granulated blast furnace slag (GGBS) with the addition of different percentages of steel fibers, which provides for high performance, durability and better serviceability in addition to overall economy in the long run.  The compressive strength studies were carried at after 7 and 28 days after curing of concrete. The workability of the different mixtures was constantly maintained by optimum usage of superplasticizers. The results showed that with the increase in the percentage of steel fibers the compressive strength of the matrix increased. The maximum strength of UHPFRC, which was achieved under laboratory conditions, was 158 MPa, after 28 days of curing. This strength was achieved for the mix wherein the overall binder content included 8% silica fume, 2% nano silica, 10% GGBS 
(with remaining 80% as cement content) along with the addition of 1.5% steel fibers. UHPC does not contain the capillary pores of traditional precast or glass-reinforced concrete, it does not absorb water and degrade in demanding freeze/thaw or marine conditions.

Key words: Ultra-High Performance Fibre-Reinforced Concrete, Ground granulated blast furnace slag, Glass-reinforced concrete