| Key Impacts of Iron Presence on Wastewater Treatment Through Subsurface Flow Constructed Wetlands: Critical Review |  |  |
| --- | --- | --- |
| Mohammad Baquir Aligarh Muslim University, Aligarh baquir.alig@gmail.com | Syed Shuja Askari School of Management Sciences, Lucknow syedshuja@smslucknow.ac.in | Sohail Ayub Aligarh Muslim University, Aligarh sohailayub@rediffmail.com |

Abstract - Treatment effectiveness in subsurface vertical flow constructed wetlands is impacted by iron cations present in wastewater. Iron influences the makeup of microbial communities, pathways for the degradation of pollutants, and its interactions with organics and pollutants enhance the elimination of impurities by adsorption and precipitation. In artificial wetlands used for wastewater treatment, iron speciation is redox-dependent and involves changes to organic contaminants, phosphorus, nitrogen, and sulfur removal. But elevated iron levels can also result in the formation of iron oxides, which decrease water movement and substrate permeability. Effective design and operation of these wetlands depend on an understanding of the range of ideal iron concentrations for effective wastewater treatment.

Keywords - Iron cations, microorganisms, redox, wastewater, subsurface constructed wetlands

# I. Introduction

Constructed wetlands (CWs) are decentralized eco-systems designed and managed to treat wastewater by mimicking natural wetlands conditions [1,2] same as the physical, chemical, and biological processes [2]. CWs have been created as an alternative to traditional centralized wastewater treatment systems in the last several decades due to their economical and environmentally advantageous method of treating wastewater [1]. From the perspective of technical application, the design and construction of CWs has expanded from conventional basic models to numerous new combinations, technological modifications, and operations to increase the performance for pollution removal [3,4]. This is in line with the growing interest in CW technology [4]. Researchers have previously demonstrated that CWs are viewed as a "black box" in the realm of wastewater treatment [5]. For instance, an extensive amount of research has been done on the microbial transformations of carbon (C), nitrogen (N), phosphorus (P), and sulfur (S). Additionally, the dynamics of new organic pollutants and heavy metals, such as antibiotics and pharmaceutical toxins, in constructed wetlands (CWs) remediation process extending sustainable application potential. The mechanisms behind these transitions have certainly become more apparent over the past few decades due to all of these evaluations [6]. Due to the fact that iron (Fe) is a necessary element, the prevention and mobility of Fe in CWs influences the geochemistry reaction of the other components in CWs in addition to taking part in numerous physiological processes including plant photosynthesis. Despite the observation of iron removal performance in certain constructed wetlands (CWs), especially those handling acid mine drainage, there is a lack of complete understanding regarding the dynamic mitigation and retention of iron in various emergent wetland layouts and intensive operations [7]. Further clarification is required about the relationship between iron cycling and the microbial transformations of all four key components and emerging pollutants in CWs. Conclusions on the fundamentals and mechanisms of the observed interactions with multiple different parameters throughout the research are hard to come by in this field of study [7,8].

The existence of Fe in diverse wastewater treated in CWs and its fundamental redox-controlled cycle in CWs with distinct setups and approaches to operation will subsequently be summed up. The relationships between the geochemical cycling of iron and the other components (C, N, P, and S) will next be covered in detail. Wetland plants' health response to Fe toxicity and deficiency in CWs created with particular treatment goals will be assessed. We will also address the function of iron plaque in the sequestration and translocation of other trace metals. Lastly, potential directions for future study on Fe in CWs will be highlighted.

# II. Iron biogeochemistry

One of the most abundant resources on Earth is iron, which comprises no less than 5% of the planet's crust. Since iron is a transition element, it can exist in a variety of oxidation states, ranging from -2 to +6, with +2 (soluble Fe2+) and +3 (insoluble Fe3+) being the most prevalent [9]. This segment will primarily elucidate the foundational aspects of ferrous biogeochemistry within the context of treatment wetlands and analogous environments. The discourse will encompass: (1) the oxidative transmutation of ferrous iron (Fe(II)) to ferric iron (Fe(III)), followed by its subsequent hydrolytic conversion to ferric hydroxide (Fe(OH)₃) or oxy-hydroxide; (2) the characteristic metabolic processes of select chemoautotrophic bacteria that precipitate hydroxides and oxides; (3) the aqueous interaction of ferric sulfate, a product of oxidation, yielding ferric hydroxide and sulfuric acid; (4) the nitrate-mediated oxidation of Fe(II); (5) the Feammox process; and (6) the chemical reduction of ferric oxide facilitated by the presence of sulfide.

# III. Fe OCCURRENCE IN SEVERAL CWs EFFLUENTS

Wetland matrix and influent wastewater are the two primary sources of Fe in CWs used for wastewater treatment. Iron is an inevitable component of many wastewaters, such as landfill or acid mine drainage, domestic and municipal wastewater, and surface runoff, due to water's inevitable interaction with underlying geologic structures [7]. In this section, the levels of iron in different wastewaters will be compiled and discussed.

# IV. Fe PREVENTION AND ACCUMULATION IN CWs

Plant uptake and the sedimentation of Fe3+ precipitates are the two main processes involved in the preventive and retention of Fe in CWs. Numerous biotic and abiotic smaller processes of the sedimentation process are dependent on the depth and water flow path along the wetland's length. This section will address the immobility of roots and the Fe uptake performance of wetland plants [8,10]; subsequent sections will address the role of plants in regulating redox potential in the rhizosphere and guiding dynamic Fe modifications in CWs through the dispersion of oxygen and other exudates. This portion also includes the facts about how heavy rain affects remobilization and seasonal and diurnal variations in Fe reduction and oxidation in the CW bed, which contributes to our knowledge of the dynamic prevention and accumulation of Fe in CWs [11].

# V. Fe CONCENTRATION LEVELS EFFECT ON PLANTS IN CWs

One crucial factor that must be taken into account for long-term operation in CWs is the condition of the wetland vegetation. One well-known element that is necessary for plant growth is iron. High quantities of iron in the reduced form (Fe2+) or low bioavailability of Fe can affect the function of CWs in wastewater treatment by causing significant health symptoms in macrophytes [12]. The effect of a shortage of iron or toxicity to wetland plants in various scenarios, including types of CWs, operating tactics, and configurations, will be the main topic of discussion in this part.

# VI. ROOT WITH IRON PLAQUE GUIDED SEQUESTRATION AND HEAVY METAL/NUTRIENT TREATMENT

The deposition of ferruginous accretion upon the radicular epidermis of botanical entities, a phenomenon of commonplace occurrence within the confines of constructed wetlands (CWs) employed for the purification of sundry effluents, may constitute a decima of the aggregate radicular mass and extend its influence up to 5–50 μm into the rhizospheric domain [13]. The acidic milieu engendered by the extrusion of protons and organic exudates from the roots serves to solubilize ferrous ions from the wetland's matricial components. The subsequent diffusion of these solubilized ferrous ions from anoxic regions to those replete with oxygen within the rhizosphere engenders the formation of this ferruginous accretion. Thus, the genesis of this plaque is a consequence of the oxidative transmutation of ferrous (Fe(II)) to ferric (Fe(III)) ions by the oxygen emanating from the roots, followed by the precipitation of ferric oxides upon the radicular surface [13,14].

The ferruginous plaque extant in natural ecosystems is generally characterized by a substantial specific surface area, attributable to its distinctive ferric phase and degree of crystallinity, and the possession of hydroxyl (-OH) functional group. These intrinsic attributes endow the plaque with the capacity to engage in reactions with diverse trace metals and nutrients, such as phosphorus, thereby exerting an influence on their biogeochemical cycles [15]. Previously, considerable scholarly endeavors have been directed towards elucidating the role of this ferruginous accretion in the sequestration of trace metals and the assimilation of nutrients, exemplified by phosphorus. Research conducted in oryza-cultivated soils and pristine wetlands has been comprehensively reviewed by the erudite works. Consequently, from the perspective of employing CWs for effluent treatment, the ensuing discourse shall elucidate and deliberate upon the potential role of this ferruginous plaque in trace metal sequestration and its interaction with phosphorus uptake [15,16].

VII. MUTUAL INTERACTIONS AMONG Fe/P/S IN CWs

## The governance of phosphorus is invariably among the most critical monitoring strategies in the process of effluent purification to mitigate the peril of eutrophication. The adsorption of phosphorus to the substratum within constructed wetlands (CWs) assumes a paramount role. The robust affinity of phosphate ions for ferric oxides engenders a preferential selection of ferruginous materials as the substratum in CWs. In particular, the judicious selection of specific industrial residua containing ferrum as substrata to augment phosphorus retention within the wetland matrix is emerging as a cardinal design and construction stratagem [2,17].

Moreover, within the well-rooted zones of a wetland matrix, there exist myriad spatial and temporal micro-scale redox gradients, a consequence of the liberation of oxygen from the radicular systems. These redox gradients in the radicular zones can concomitantly facilitate intricate interactions between diverse processes, as an illustration, ferrum-phosphorus sequestration and phosphorus release, ferrous reduction, and/or sulphate reduction [2,5,6,17]. The presence of sulphate may not be significantly problematic in CWs. However, the synergistic biogeochemical cycling of ferrum and sulphur, driven by redox dynamics, has been demonstrated to influence both the availability and mobility of phosphorus in CWs. The subsequent discourse shall primarily elucidate the multifarious interactions of ferrum, phosphorus, and sulphur within the context of constructed wetlands [2,5,17].

VIII. ROLE OF Fe INTERACTIONS IN N REMOVAL IN CWs

## Iron cycling is very important to the nitrogen biogeochemical cycle, even though it only involves the rotation of two primary chemical valences. Different nitrogen transformations were frequently combined with the oxidation and reduction of Fe. The importance of Fe cycling may be greater in situations when there are little organic carbon sources [17,18]. The processes of ammonium oxidation connected to dissimilatory reduction of iron oxides under anaerobic circumstances (Feammox) and denitrification based on nitrate-dependent Fe(II) oxidation will be mostly explored in this section.

IX. PROSPECTIVE FOR FURTHER RESEARCH STUDIES

Future investigations and developments on the cycling of iron and its multiple interactions will be addressed here because of the intricacy of different wastewater contaminants and micro-redox differences in the root and rhizosphere in CWs.

References

[1] H. Wu et al. “A review on the sustainability of constructed wetlands for wastewater treatment: design and operation,” Bioresource technology, vol. 175, pp.594–601, 2015.

[2] J. Vymazal, “Constructed Wetlands, Surface Flow,” Encyclopedia of Ecology, Academic Press, pp. 765–776, 2008.

[3] L. E. Fernández Ramírez et al. “Technological Innovations in the Application of Constructed Wetlands: A Review,” Processes, vol. 11, no.12, pp.3334, 2023.

[4] J. L. G. Kumar and Y. Q. Zhao, “A review on numerous modeling approaches for effective, economical and ecological treatment wetlands,” Journal of environmental management, vol. 92, no. 3, pp.700–406, 2011.

[5] G. Langergraber, “Process Based Models for Subsurface Flow Constructed Wetlands,” in Water and Nutrient Management in Natural and Constructed Wetlands, J. Vymazal, Ed. Springer, Dordrecht, 2010, pp. 21–35.

[6] M. Shirdashtzadeh et al. “Microbial communities and nitrogen transformation in constructed wetlands treating stormwater runoff,” Frontiers in Water, vol. 3, pp.751830, 2022.

[7] S. Wu et al. “Critical review: biogeochemical networking of iron in constructed wetlands for wastewater treatment,” Environmental science & technology, vol. 53, no.14, pp.7930–7944, 2019.

[8] Y. Ma et al. “Iron scraps enhance simultaneous nitrogen and phosphorus removal in subsurface flow constructed wetlands,” Journal of hazardous materials, vol. 395, pp.122612, 2020.

[9] K. Armstrong et al. “Deep magma ocean formation set the oxidation state of Earth’s mantle,” Science, vol. 365, pp.903–906, 2019.

[10] P. F. Schwammberger et al. “Nutrient uptake by constructed floating wetland plants during the construction phase of an urban residential development,” The Science of the total environment, vol. 677, pp.390-403, 2019.

[11] Hui‐Ting Huang et al. “Free Radicals Produced from the Oxidation of Ferrous Sulfides Promote the Remobilization of Cadmium in Paddy Soils During Drainage,” Environmental science & technology, vol.55, no.14, pp. 9845–9853, 2021.

[12] S. Kumar et al. “Removal of nutrients from domestic wastewater using constructed wetlands: assessment of suitable environmental and operational conditions,” Environmental Sustainability, vol. 3, no.4, pp.341-352, 2020.

[13] M. Maisch et al. “From plant to paddy—how rice root iron plaque can affect the paddy field iron cycling,” Soil Systems, vol. 4, no.2, pp.28, 2020.

[14] M. Zhu et al. “Precipitation pathways for ferrihydrite formation in acidic solutions,” Geochimica et Cosmochimica Acta, vol. 172, pp.247–264, 2016.

[15] E. D. Swanner et al. “The biogeochemistry of ferruginous lakes and past ferruginous oceans,” Earth-Science Reviews, vol. 211, pp.103430, 2020.

[16] X. Wan et al. “Effect of iron plaque on the root surface of hydrophyte on nitrogen and phosphorus transformation,” Bioresource Technology Reports, vol.12, pp.100566, 2020.

[17] H. Bacelo et al. “Performance and prospects of different adsorbents for phosphorus uptake and recovery from water,” Chemical Engineering Journal, vol. 381, pp.122566, 20220.

[18] A. Kappler et al. “An evolving view on biogeochemical cycling of iron,” Nature Reviews Microbiology, vol. 19 (2021): pp.360 – 374, 2021.

# About the Authors

Mohammad Baquir is a PhD student in Environmental Engineering in Aligarh Muslim University, Aligarh, India. He has done M.Tech (Environmental Engineering) in 2012 and B.Tech (Civil Engineering) in 2009 both from the same university.

Syed Shuja Askari is an Assistant Professor at SMS, Lucknow in Civil Engg. Department. Currently he is doing PhD in Environmental Engg. (part time) from Integral University. He has done M.Tech (Environmental Engineering) in 2017 and B.Tech (Civil Engineering) in 2012.

Sohail Ayub is professor at Aligarh Muslim University, Aligarh. He is life member of institution of engineers, Institution of Public Health Engineer (India) and Institute of Standards Engineers. His research interests and publications are in the area of water, wastewater, solid waste and air pollution. He has about 100 research papers published in reputed national / international journals and presented in conferences.